## BIO373L   			 Using a Hobo Data Logger

## Gilbert/Plowes

## Programming HOBO

- plug-in the Hobo to the adapter connected to the computer

- open Boxcar Pro program on the desktop

- open Logger menu and select “Launch”;  choose desired settings (time interval, channels, time delay). Remember to give the Hobo a distinctive Name.

- click “Launch” to program Hobo.  Once the upload is complete, disconnect the Hobo and check that the led is flashing slowly.

5)  you’re good to go

# Downloading data from HOBO

- plug in Hobo to adaptor connected to the computer

- open Boxcar Pro program on desktop

- open Logger menu and select “Readout”

- A window will appear showing a figure (most likely of temperature).  To view another climatic variable (such as RH, Temp C, external probe temp., etc…), click the left-most box above the figure.

- Open File menu and select “Export as text.”  Give your file a name and have it saved to the desktop.

- Open MS Excel

- Open the File menu and select “Open”; then specify your text file that is saved on the desktop.

- Select “Finish” and your data should appear in an excel spreadsheet